============================

** BRINGING UP THE SYSTEM **

============================

INTRODUCTION

------------

The following intructions should be followed when installing an XCOMP

ST/S or ST/R controller with a MiniScribe MS-3012 Hard Disk Drive.

INSTALLATION

------------

1. Set the controller I/O address so that it does not conflict

with any other I/O in your computer. XCOMP uses I/O addresses

70H-77H for the S-100 Controller (ST/S) and 80H-87H for the

Ribbon Cable Controller (ST/R). Standard switch  settings

are shown below.

SWITCH	S-100	R/C

------	-----	----

1	 OFF	OFF

2	 ON	ON

3    ON	ON

4    ON	ON

5    OFF	ON

6	 ---	---	* Not used...can be either ON or OFF

7	 OFF	OFF

Consult the appropriate XCOMP Users Manual if want to set the switches

(I/O Address) to some other range. Make sure that switch 7 is always set

to the OFF position -- this enables Buffered Step Mode.

2. Change the equate statement CBASE in all programs to reflect the I/O

address used in Step #1. Note that there are two CBASE statements in

each program -- one for S-100 & one for Ribbon Cable. Assemble all the

programs using the Digital Research macro assembler MAC.

3. Install the controller in your system. Do so with power off!

4. Connect the short 26 pin ribbon cable between the two boards.

5. Connect both the 34 & the 20 pin cables from the controller

to the drive. Pin 1 of both cables on the controller are

connected to Pin 1 on the drive connectors. Pin 1 is denoted

by a triangle on the connectors and by the red wire on the

ribbon cable.

6. Plug the power cable into the drive. Do so with power off.

TESTING

-------

Run the following programs in sequence:

1. MHBT. This will test the buffer memory of the controller. It also

ensures proper I/O address decoding and the ability of the host

processor to transfer data to/from the controller.

2. MHATS. Tests the step logic of both the controller & the drive.

Also ensures that the control cable (34 pin) is installed

correctly.

3. MHFMT. Formats the drive. This needs to be done before data can

be written onto the drive. Tests the ability of both the controller

& the drive to read & write data. Ensures that the data cable (20 pin)

has been installed correctly. Note that this program DESTROYS any data

already on the disk.

4. MHRDT. Random read/write test. Good test to verify that the complete

sub-system (including drive power supply) is working properly. Note

that this test DESTROYS any data already on the disk.

5. At this point you are ready to bring up CP/M on your hard disk.

################################################