=====================

** Back End Driver **

=====================

INTRODUCTION

------------

MHBED is a hard disk only back end I/O driver for CP/M 2.2 based systems.

MHBED gives the user the ability to attach hard disk to a current system

without any alteration to the user's CBIOS. This is accomplished by moving

the current CP/M system down 2K in memory; the back end driver is then loaded

into the free space made available at the end of memory. Once loaded, MHBED

alters the user's CBIOS Jump Table to point to itself. It then arbitrates

between calls for the floppy disk and calls for the hard disk.

CONFIGURING THE DRIVER

----------------------

Lets assume that you are currently running a 64K floppy system and you

want to use MHBED. Using directions supplied by Digital Research or the

manufacturer of your system, build a 62K CP/M system. This opens up 2K of

RAM for the back end driver. In the program MHBED.ASM alter the equate

statement STRT to read  "STRT  EQU  62*1024". In the program MHMDC.ASM alter

the equate statement TADR  to read "TADR  EQU  62*1024". Also set the

equate statement "MAXFD" (in MHMDC.ASM) to the number of floppy disk drives

in your system. Assemble both MHBED and MHMDC. Use DDT to create a COM file

called BED.COM; its done like this:

A>DDT MHMDC.HEX		;Load DDT & MHMDC (@ 100H) into RAM

-IMHBED.HEX

-RC00			;Read MHBED into RAM starting @ 400H

-G0			;Warm Boot CP/M

A>SAVE 8 BED.COM	;Create the COM file

NOTE: If you build the backend driver at some location other

than 62K you will to change the value used in the "R"

command above. The value to be used may be found in

MHBED. After assembly look for the equate statement

"OFFSET" near the beginning of the program. "OFFSET"

will give you the value to be used in "R".

USING THE DRIVER

----------------

Boot your floppy disk system as you would normally. Execute the file

BED.COM by typing: "A>BED <CR>". This will load the driver code. The hard

disk becomes the next drive in line; i.e., if you are currently using

two floppy disks as drives A: & B:, the hard disk will become drives C:

& D:.

###################################################################################