================

** SYSBLD.DOC **

================

The following instructions are designed to help you build a

62K CP/M system.

STEP #1. Use MOVCPM to create a COM file with the basic CP/M

system in it. Call it CPM62.COM.

STEP #2. Alter MHIOS to fit your specific hardware configuration.

Assemble MHIOS and produce an object file MHIOS.HEX.

STEP #3. Repeat STEP #2 for MHBOOT and MHPUT.

STEP #4. Put all the pieces together to form a complete system.

Save that system on your diskette. It is done thusly:

A>DDT CPM62.COM		;Load DDT and BDOS

DDT VERS 2.2

-IMHPUT.HEX

-R			;Read MHPUT into RAM at 100H

-IMHBOOT.HEX

-R880			;Read MHBOOT into RAM at 880H

-IMHIOS.HEX

-R2D80			;Read MHIOS into RAM at 1F80H (See NOTE below)

-G0			;Warm Boot CP/M

A>SAVE 40 MH62.COM	;Save the system on your diskette

NOTE: If you assemble the I/O driver at some location other

than 62K you will to change the value used in the "R"

command above. The value to be used may be found in

MHIOS. After assembly look for the equate statement

"OFFSET" near the beginning of the program. "OFFSET"

will give you the value to be used in "R".

STEP #5. Assemble the disk format program MHFMT. Use it to format

ths disk drive.

STEP #6. Write the system onto the systems area of the drive. It is

done like this:

A>MH62

STEP #7. Boot the system and test it.

NOTES

-----

1. Because of the size of MHIOS, the system requires an additional 2K

bytes of memory. In other words, if you have 64k of memory you'll

need to build a 62K system.

2. For those familiar with CP/M V1.4, the SYSGEN memory layout for BOOT,

BDOS, & CBIOS has changed. BOOT starts 80H bytes lower in RAM because

the sector size on the hard disk is 256 bytes in length (as opposed to

a floppy disk which has a sector length of 128 bytes).  CBIOS starts

100H bytes higher in RAM because BDOS has grown.

BOOT	BDOS	CBIOS

1.4	900H	980H	1E80H

2.2	880H	980H	1F80H

##############################################################################################################