================

** MiniScribe **

================

This is the initial release of CP/M 2.2 support software for an XCOMP ST/S

(ST/R) Hard Disk Controller used in conjunction with a MiniScribe MS-3012

Hard Disk Drive.

All the software on the diskette was written using the Digital Research

macro assembler MAC.

The following programs are provided on the diskette:

MHATS -- Tests the seek logic of both the controller and

the drive.

MHBED -- Back end driver (hard disk only).

MHBOOT -- Used in conjunction with MHROM to boot CPM from

the hard disk.

MHBT -- Tests the buffer memory of the controller.

MHFMT -- Used to format the hard disk. Does surface analysis &

sector sparing.

MHGET -- COM file used to boot the system from the hard disk.

MHIOS -- CPM 2.2 I/O driver.

MHMDC -- Used to move the back end driver (MHBED) code to its

target address.

MHOFF -- Used to put the heads over the landing zone for shipmemt

or at the end of the day before power off.

MHPUT -- Used to write the system onto the hard disk.

MHROM -- Example of ROM bootstrap for the hard disk.

MHRDT -- Random read/write diagnostic test.

MHVER -- Used to read (verify) the disk.

Two of the support programs (MHROM & MHGET) do direct console I/O.

These programs will require modification to run properly in your system.

A number of the programs report errors. Some of the error messages

include information about the drive and controller status and the place

(on the disk) where the error(s) occurred. Abbreviations are used and the

the data is printed in hex. Abbreviations used are:

DST = Drive Status

CST = Controller Status

CYL = Cylinder

HD = Head

SECT = Sector

If you choose to use a set of I/O addresses other than 70H-77H (S-100)

or 80H-87H (Ribbon Cable) you'll need to change the equate statement CBASE

in each program. Note that there two CBASE statements in each program: one

if you are using an S-100 Controller (ST/S) & the other if you are using a

Ribbon Cable Controller (ST/R).##############################