Introduction

CP/M Disk File Structure

Install Disk Doctor

Invoking Disk Doctor

Recovering Disks and Files, Wards A,B, and C

Un-erasing Files, Wards D and E

Common Disk Problems and Recovery Procedures

:Introduction

Disk Doctor is comprehensive diskette  recovery  package

comprised of an installation program (INSTALL.COM) and  five

recovery procedures (DOCTOR.COM),  refered  to  as  "Wards,"

each capable of performing a specific recovery task.   Three

general kinds of facilities are provided.  Ward A  can  help

restore the usefulness of disks with malfunctions  in  areas

not assigned to files.  Wards B and C can aid in  recovering

files affected by disk malfunctions.  Wards D and E can  aid

in recovering  files  accidentally  erased.    As  an  added

benefit, Ward A can also be used to certify  new  disks  and

eliminate any bad sectors before they  can  cause  problems.

TWO IMPORTANT WARNINGS

If you know or suspect  that  you  have  both  unwanted

erasures and malfunctions on the same disk,  you  must  deal

with all erasures first.  This is  necessary  because  those

procedures  used  to  recover  from  disk  malfunctions  may

overwrite erased files, making  their  recovery  impossible.

Before  attempting  to  use  any   of   DISK   DOCTOR's

facilities,  you  must  be  certain  that  it  is   properly

installed for each disk format on which you  intend  to  use

it.  DISK DOCTOR comes pre-installed for the type  of  media

on which it was supplied; but if you wish to use it on other

media or densities,  a  careful  reading  of  Chapter  3  is

recommended.

: CP/M  FILE STRUCTURE

This system is hierarchical in structure.   Its  levels

are, in order of decreasing size:    the  disk  itself,  the

file, the extent, the track, the group, and the sector.   It

is one of the most convenient and desirable features of CP/M

that all ordinary data manipulations  may  be  performed  in

terms  of  disks  and  files  alone.    However,  to  better

understand how DISK DOCTOR's Wards perform their tasks  and,

in particular to make good use of Ward B, you  will  need  a

good grasp of all levels of the CP/M  file  structure.    In

this chapter we will describe each level  of  the  hierarchy

starting with the smallest sized structure and  working  up.

The smallest unit (above the byte--or character) in the

hierarchy is the sector.  A sector is the  minimum  unit  of

disk data accessible by CP/M.  CP/M defines a sector as  128 bytes  (characters).    It  expects  these  sectors  to   be

organized into tracks.  The  track  is  merely  a  means  of

organizing and locating the individual sectors  on  a  disk.

The unit in which CP/M allocates sectors  to  files  is

the group.  A  group  is  composed  of  a  fixed  number  of

sectors, either 8 or 16 depending on the  implementation  of

CP/M on your system.  Space for the disk directory  is  also

allocated in groups.

An extent is, in turn, made up of a variable number  of

groups with the maximum number of groups per extent being  8

or 16 depending on your CP/M  implementation.    (Most  CP/M

systems have a maximum of 128 sectors per extent:  either 16

groups of 8 sectors,  or  8  groups  of  16  sectors.    The

Intertec Superbrain, however, has extents with a maximum  of

16 groups of 16 sectors or 256 sectors per extent.)  A  CP/M

file  is  composed  of  one  or  more  extents;   therefore,

assigning  a  variable  number  of  groups  to   an   extent

facilitates  the  efficient,  dynamic  allocation  of   disk

storage to files.

A disk may contain from zero to many files.  Each  disk

is accessed by the system through its assigned  drive  name.

A disk retains its hierarchical structure with the exception

of the drive name when it is removed from the drive.    Thus

it makes sense to talk about a disk containing a file,  even

if the disk is not in a drive (i.e., is not assigned a drive

name).

FORMAT OF A DIRECTORY ENTRY

Thus far, we have described the structure of CP/M  disk

files themselves.  However, for each extent  of  each  file,

there exists, also on  disk,  a  32-byte  field  called  the

directory entry.  All directory entries together make up the

disk directory which usually resides within the first one or

two groups on the disk.   A  directory  entry  fulfills  two

important functions in the CP/M disk file system:  first, it

provides a means of managing the storage allocated to  files

on the disk; and second, it provides the means  of  locating

those same files.

Knowledge  of  the  format  of  a  directory  entry  is

potentially useful, since Ward B (in conjunction with CP/M's

DUMP or DDT facilities) can be  used  to  examine  directory

entries (see p.  16), and since sophisticated use of Ward  B

requires their examination.   The  diagram  appearing  below

represents the format of the 32-byte field  of  a  directory

entry.

0                      16                    31

|                       |                     |

|XN|NN|NN|NN|NT|TT|EX|XS|GG|GG|GG|GG|GG|GG|GG|GG|

N -- byte in filename field

T -- byte in file type field

E -- extent number

S -- extent size in sectors

G -- byte in group allocation map field

X -- byte of no interest to us

The group allocation  map  field  contains  a  list  of  the

logical group numbers in the order the groups were  assigned

to the file.  Three formats in  use  (under  different  CP/M

implementations) for this 16-byte field are  represented  in

the three diagrams appearing below (the 'g's stand for group

numbers and the '0's stand for zero bytes --recall that 8080

word format is low order byte, high order byte).

16                    31

|                     |

|GG|GG|GG|GG|00|00|00|00|

8  groups/extent,  group  number   stored   per   byte

16                    31

|                     |

|G0|G0|G0|G0|G0|G0|G0|G0|

8  groups/extent,  group  number   stored   per   word

16                    31

|                     |

|GG|GG|GG|GG|GG|GG|GG|GG|

16  groups/extent,  group  number  stored   per   byte

:  HOW TO INSTALL DISK DOCTOR

DISK DOCTOR is composed of two files:   DOCTOR.COM  and

INSTALL.COM.  The first few sectors  of  DOCTOR.COM  contain

information about the disk environment in which  it  expects

to operate.  It is this information that INSTALL.COM  alters

to tailor DOCTOR.COM to a particular new disk format.  To do

this  INSTALL.COM  requires:    that  you  insert   a   disk

containing DOCTOR.COM in drive A and a blank formatted  disk

in a drive which you may specify when prompted (CR  defaults

to B), and that you supply it with  the  proper  values  for

three parameters when you are prompted  to  do  so.    These

parameters are:

1.) the lowest physical sector number on a track (either  0

or 1, most drives start numbering at 1);

2.) the number of sectors on a track (in CP/M  version  2.0

and later STAT <drive name>:DSK:  provides  the  number      of sectors on a track);

3.) the number of tracks on a disk.

: Invoking Disk Doctor

DISK  DOCTOR  exists  as  an  executable  command  file

(DOCTOR.COM) on disk.  It does not require any command  line

arguments when invoked.  It currently does  require  a  dual

disk drive system to perform its functions.  There should be

a disk containing DOCTOR.COM in drive A before invoking DISK

DOCTOR.  Once DISK DOCTOR is  loaded  into  memory  and  the

first question answered, that disk can be  removed,  as  the

program resides in memory until exited.

First DISK DOCTOR asks you to select one of three  disk

operation display options.  To enable this display is useful

only if your console can support a carriage return without a

line  feed.    When  the  program  is  performing  disk  I/O

operations, it will display the track, group, and sector  of

each operation by rewriting  the  same  line  with  updates.

When bad data is encountered a carriage return  and  a  line

feed are executed--leaving  the  guilty  track,  group,  and

sector displayed above the current line.  Choosing the first

option, "S", will result in a full display of all  disk  I/O

operations by sectors.

:  Recovering Disks and Files:  Wards A, B, and C

The basic principle underlying Wards A,  B,  and  C  is

that most disk malfunctions  are  simply  the  result  of  a

single sector failing to be read or written properly.   Such

a malfunction will have different effects and  will  require

different remedies depending on whether the  bad  sector  is

within a group assigned to the directory or  to  a  file--or

within a group not presently assigned.

BAD SECTORS IN GROUPS ASSIGNED TO THE DIRECTORY

If there is a single bad sector in the directory  of  a

disk, CP/M will not read the  directory  past  that  sector.

Hence, although some files may still be read,  none  may  be

written.  In most cases, the bad sector will have nothing to

do with the file or files of  interest  to  the  user.    To

placate this protective feature of CP/M, Ward C will copy an

entire disk, replacing all data in any  damaged  sectors  in

the directory with the disk formatting character, 0E5h; thus

allowing the rest of the directory to  be  accessed  through

the usual system commands.  If the bad  sector  does  affect

the directory entry of a  file  of  interest,  then  Ward  B

provides a means of working around  the  system  to  extract

data from the directory and the file.

BAD SECTORS IN GROUPS ASSIGNED TO FILES

If a file contains a single bad sector, CP/M  will  not

read the file beyond that sector  and  the  entire  file  is

rendered useless.  While a sector of bad data in  an  object

file might mean that the  file  is  useless,  a  single  bad

sector in a source or  text  file  would  be  only  a  minor

annoyance if the rest of the file could be recovered.   Ward

C will, as it copies, also  replace  all  data  in  any  bad

sectors that it finds in files  with  ASCII  spaces.    This

allows the user to edit the file and easily repair it.    It

might seem that a file could be recovered down to  a  single

bad byte, but this is not possible because many disk control

systems buffer their read operations (read data  in  blocks)

and most errors cause a loss of sychronization.

BAD SECTORS IN UNASSIGNED GROUPS

If a bad sector appears in an  unassigned  group,  CP/M

will not make use of any of the good sectors above  it,  and

will refuse to close any file that attempts to write to  the

bad sector.  Since each disk storage operation will  attempt

to use the lowest available  group,  the  disk  will  be  as

useless as if it were full.  Ward A  circumvents  this  CP/M

protective  feature  by  means  of  assigning   all   groups

containing bad sectors to a special file intended  never  to

be used again.

WARD A

This Ward is concerned with restoring the usefulness of

a disk rather than recovering data from it.  Like the  other

DISK DOCTOR programs, it uses the most primitive BIOS  hooks

to bypass the automatic CP/M features.   Ward  A  loads  the

directory from the disk containing  the  bad  sector(s)  and

searches for all unassigned groups.  This prevents damage to

the intact files on the disk.    Each  unassigned  group  is

written and read to validate its integrity.  If the  program

locates a bad  sector,  it  will  assign  the  entire  group

containing  it  to  a  special  file  called  MORGUE;   thus

effectively  eliminating  the  bad  group  from  the   disk.

Obviously, you should never use the filename MORGUE  in  any

of your own programs.  Also, unless you want  the  same  bad

groups to return to haunt you, you must NEVER  ERASE  MORGUE

FILES.  (Under CP/M Version 2.0 and later, MORGUE files have

the R/O attribute set to prevent such erasures.)

The file type (extension) of a MORGUE file also  serves

a purpose.  The first time Ward A is run on a  disk;  if  it

finds bad sectors, it will open a file:  MORGUE.NO1.    Each

time it is rerun on the same disk;  if  it  finds  more  bad

sectors, it will  generate  another  consecutively  numbered

MORGUE file (e.g., MORGUE.NO2).  Thus it is possible to  see

at a glance if a disk has been prone to trouble,  and  avoid

selecting it for critical work.  Sometimes it  is  necessary to run Ward A on a disk several times in order to cause  all

faulty sectors to fail.

The three "do's and don'ts" listed below are  essential

to the effective use of Ward A.

1.) It cannot be overemphasized that you must  never  erase

any MORGUE files.    Erasing  a  MORGUE  simply  undoes

everything Ward A has done.

2.) If there are erased files on a disk which you  wish  to

recover  by  using  Ward  D,  you  must  do  so  before

executing Ward A on the disk.  Once it is executed,  no

erased file can be recovered as all of its groups  will

be overwritten.

3.) Before running Ward A  on  a  disk,  all  temporary  or

aborted files (such as those with the  extension  .$$$)

on the disk should be erased.  Some  text  editors  may

close such files when bad sectors are encountered,  and

then re-open them later.  This will  cause  Ward  A  to

miss the troublemaker, since the  group  containing  it

appears to be assigned to a file.

If you select Ward A from DISK DOCTOR's menu, you  will

first be prompted to place your bad disk  in  the  "patient"

drive you previously designated.  You will then be given one

last chance to decide whether you actually want to run  Ward

A or not.  If not, the program returns you  to  "Admitting."

WARD B

This Ward allows  you  to  pull  arbitrarily  specified

groups or sectors of data off  of  one  disk  (patient)  and

collect them in a specified file on another disk (recovery).

You can then edit this file, discarding the data that is  of

no value  to  you.    This  is  a  powerful,  though  rather

primitive capability.  To get the maximum  benefit  from  it

requires an informed and thoughtful approach.    You  should

note that, through this Ward, the directory is as accessible

as the files.  If you are searching for the groups  assigned

to a particular file; examining the directory, even if it is

damaged, should provide valuable information.

If you select Ward B from DISK DOCTOR's menu, you  will

first be prompted to place the bad disk  in  the  previously

specified  "patient"  drive  and  a   good   disk   in   the

corresponding "recovery" drive.  You will then  be  prompted

to specify the collection file on your recovery disk.  After

you have typed your collection filename and hit return,  the

program will ask you whether you wish  to  recover  data  by group or by sector.  Then according to your choice, data  is

collected into a  buffer  by  specifying  either  hex  group

numbers or hex track and sector numbers when  prompted.    A

bad disk is not required to have a good directory as  it  is

never accessed unless specified for collection.   In  either

collection mode, a carriage return entry will cause the next

logical group or track-sector combination to  be  collected.

When all desired data has been collected, the  file  can  be

closed by typing a period and carriage return (same  as  DDT

Substitute).  If the buffer becomes full, a message will  be

displayed, and the file will be closed  automatically.    In

the case of very long files, several recovery files  may  be

specified, and then concatenated using PIP.

Files recovered using Ward B are just  like  any  other

files under CP/M.  Files containing text may be viewed using

the TYPE transient.  With enough patience, text  files  with

directory entries damaged beyond  recovery  can  be  gleaned

from a disk.  Files containing object code may  be  examined

using DDT dump or DUMP transients.    File  directories  may

also be examined by these means.  Much information  that  is

not otherwise visible can be accessed through this  program.

A good example of this is the use of  Ward  B  to  determine

whether  or  not  DISK  DOCTOR  itself  has  been   properly

installed.

WARD C

This Ward is your main resource for  dealing  with  bad

sectors in files and directories.  It can best be thought of

as a very tolerant disk copying routine.  Ward C copies each

good sector, in sequence, just as it finds it; but each  bad

sector is filled with characters which function  as  blanks.

In the  case  of  directories,  the  blank  disk  formatting

character, 0E5h is used.  Bad sectors in  files  are  filled

with ASCII spaces.  This will solve most problems  involving

accessing files.  The only access problem it cannot solve is

that of an important file with a  damaged  directory  entry.

Files with directory entries unaffected by bad  sectors  can

be accessed and manipulated in all  the  ways  CP/M  usually

allows.  They can thus be examined and any  damaged  sectors

repaired to the extent possible.

One disadvantage of Ward C is that it is slow.  To copy

a disk using this program can take up to an  hour  depending

on the drive and the number of bad sectors encountered.   It

will more commonly take about ten minutes.   This  routine's

slowness results from its circumvention of  the  usual  CP/M

file handling utilities.

:  Un-erasing Files:  Wards D and E

Files may be erased in error either  directly  by  user

command (ERA) or indirectly as a result of a program's  file

handling.  Wards D and E will  allow  the  user  to  recover

completely from both  kinds  of  errors  provided  that  the

groups originally assigned to the erased files have not been

reallocated to other  files.    If  some  groups  have  been

reallocated,  it  may  still  be  possible  to  salvage  the

remaining groups in the file through skillful use of Ward B.

If no disk write operations have been  performed  since  the

erasure, then the erased file will  always  be  recoverable.

Wards D and E rely on the fact that CP/M, in  its  file

management, does the minimum amount of processing necessary.

Hence, CP/M's erasing process involves  a  few  flag-setting

operations rather than any overwriting of a file's  assigned

groups or directory entries.  This allows files with  groups

not yet reallocated  (and  thus  overwritten)  to  be  fully

recovered  by  re-setting  those  flags  to  their  original

values.

If you know the exact name of the erased file, you  can

go directly to Ward D to recover it.  If you do not know the

name, Ward E provides a directory  of  all  the  recoverable

erased files (as well as all the active files), by  extents,

on a disk.

WARD D

This  Ward  performs  its  functions  by  reading   the

directory of a  disk  and  then  rewriting  it,  with  minor

changes, back to the same disk.  The program operates in two

modes:  as an erased file recovery routine and  as  a  fast,

but dangerous directory repair routine.

The primary use of Ward D is to recover  erased  files.

After you enter the program, you will be asked to place  the

disk with the  erased  file  in  the  previously  designated

patient drive, give the unambigous filename  of  the  erased

file, and hit return.    The  program  will  then  read  the

directory of the disk and search for the specified file.  It

will indicate if it has found the file and, if so, will then

proceed to check whether any active  file  has  cannibalized

the groups of the desired file.  If any of the groups appear

in the directory entry of an active file, the  program  will

inform you that the file is not recoverable.  If not it will

un-erase the file.  If only a few groups have been  lost  to

active files, then it may be worthwhile to  use  Ward  B  to

recover  the  groups  remaining.    (NOTE:    If  many  disk

operations have been carried out since the erasure, there is

a chance that groups have been used  by  a  file  which  has

itself been subsequently erased.  If so, Ward D may not warn

you that  the  reactivated  file  is  essentially  useless.)

Ward D can recover multiple extent files, but  only  if

the following conditions are satisfied:

1.) all groups in all extents are unallocated,

2.) all extents are in ascending order,

3.) no extents are missing.

If any one of these conditions is not met, Ward D  will  not

recover the file.  Ward B is then  the  appropriate  remedy.

(NOTE:  You must rename any active file of the same name  as

the erased file you wish to  recover,  prior  to  attempting

recovery.  Erasing the namesake would  only  result  in  its

being un-erased.)

WARD E

This is a  specialized  directory  program.    It  will

display all the files on a disk; with the erased, apparently

recoverable  files   enclosed   in   parentheses:      e.g.,

(MUGWUMP.TXT).  As in Ward D, the appearance of  a  file  as

recoverable does not guarantee that it has not been  damaged

by a file which has in turn been erased.   This  program  is

very useful in searching disks for a recoverable version  of

a lost file.  Ward D would  then  be  used  to  attempt  the

recovery, as Ward E only reads the directory  and  does  not

modify it in any way.  It requires that you place  the  disk

to be searched in the designated patient drive and give your

consent for the program to proceed.

In order for a file to be recovered, it must show up in

all its extents during Ward E, and the extents  must  be  in

ascending order.  If, for example,  extent  00  is  missing,

Ward D will not attempt to recover extent  01  of  the  same

filename.  If extent 01 preceeded extent 00 in  the  Ward  E

display, WARD D would not attempt to recover  either  extent

because that version of extent 01  must  have  been  from  a

different generation of  the  file  (CP/M  always  puts  the

extents in ascending order).  Ward B can be  used  to  piece

together such fragmented files if necessary.

:  Common Disk Problems and Recommended Recovery Procedures

_____________________________________________________________

I.    File(s) accidentally   |  Invoke DISK DOCTOR (p. 10),

erased, filename(s)  |  go to Ward D (p. 19).  If

known.               |  portions of erased file(s)

|  have been reallocated, you

|  can go to Ward B (p. 16) to

|  salvage the remaining groups.

-----------------------------|-------------------------------

II.   File(s) accidentally   |  Invoke DISK DOCTOR (p. 10),

erased, filename(s)  |  go to Ward E (p. 19). Disk

unknown              |  directory with filename(s) of

|  recoverable erased file(s)

|  in parentheses will be dis-

|  played. Continue as in I

|  above.

-----------------------------|-------------------------------

III.  All existing files are |  Invoke DISK DOCTOR (p. 10),

intact, but attemp-  |  go to Ward A (p. 15).  {This

ting to create a new |  same procedure may also be

file results in CP/M |  used to screen new disks and

error message--BDOS  |  eliminate malfunctioning

ERR ON d:BAD SECTOR. |  groups from further use.}

-----------------------------|-------------------------------

IV.   An attempt to access   |  Invoke DISK DOCTOR (p. 10),

an existing file re- |  go to Ward C (p. 18). The de-

sults in CP/M error  |  sired file should be acces-

message--BDOS ERR ON |  sible on the disk copy gene-

d:BAD SECTOR, but    |  rated by Ward C, although

file directory is    |  data in any bad sectors will

still accessible.    |  be replaced by ASCII spaces.

-----------------------------|-------------------------------

V.    Any attempt to access  |  Invoke DISK DOCTOR (p. 10),

directory (such as   |  go to Ward C (p. 18). If the

entering DIR) re-    |  desired file still cannot be

sults in CP/M error  |  accessed on the disk copy

message--BDOS ERR ON |  generated by Ward C, go to

d:BAD SECTOR.        |  Ward B (p. 16) to salvage

|  what data you can from the

|  original disk or the copy.

-----------------------------|-------------------------------

TO THE USER:  As the creator of a program cannot always con-

ceive of all uses to which it may be put;  if you discover

other ways to use DISK DOCTOR to your advantage, please call

or write us at SuperSoft to tell us about it.