This document details the files changes necessary (by module) to enable continuous LBA addressing (No Holes). Please note that at this time only 64 sectors/track > LBA are supported with a maximum drive size of 8MB, this is ideally suited to our S-100 Z80 systems using CF flash drives.

No warranty is given or implied and you use this software at your own risk (Hobbyist warranty )

|||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||

==================================================================================PLEASE NOTE  - the file HSYSGEN.ASM has not had it’s wrlba: section updated as it was not my intention to write to the boot sector in this way.

==================================================================================

|||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||

==================================================================================

CPMLDR

==================================================================================

Changes to HLDRBIOS.ASM in order as listed

---------------------------------------------

Insert replacement wrlba: routine

---------------------------------------------

;  Convert CP/M Track and Sector requests to LBA and write to drive registers

wrlba:

LHLD	@TRK		;Get CPM requested Track Hi&Lo

MVI	H,00H		;zero high track byte

MOV	A,L		;load low track byte to accumulator

CPI	00H		;check for 0 track and skip track loop

JZ	lbasec

MVI	B,06H		;load counter to shift low track value 6 places to left i.e X 64

lbatrk:

DAD	H		;Add HL to itself 6 times to multiply by 64

DJNZ	lbatrk		;loop around 6 times i.e x 64

lbasec:

LDA	@SECT		;Get CPM requested sector

ADD	L		;Add value in L to sector info in A

JNC	lbatot		;If no carry jump to lba ‘total’ calculation

INR	H		;carry one over to H

lbatot:

MOV	L,A		;copy accumulator to L

;HL should now contain correct LBA value

MVI	D,0		;Send 0 for upper cyl value

MVI	E,REGcylinderMSB

CALL	IDEwr8D	;Send info to drive

MOV	D,H		;load lba high byte to D from H

MVI	E,REGcylinderLSB

CALL	IDEwr8D	;Send info to drive

MOV	D,L		;load lba low byte to D from L

MVI	E,REGsector

CALL	IDEwr8D	;Send info to drive

MVI	D,1		;For now, one sector at a time

MVI	E,REGseccnt

CALL	IDEwr8D

RET

--------------------------------------------------------------------

Updated disk parameter block to specify 64 sectors

--------------------------------------------------------------------

; IDE HARD DISK PARAMETER BLOCK:

HD$DPB:	 DPB	512,64,256,2048,1024,1,8000H

==================================================================================CPM3.SYS

==================================================================================

Changes to HIDE3.ASM in order as listed

----------------------------------------------------

Updated number of sectors to  40H (64)

----------------------------------------------------

MAXSEC		EQU	40H	;Sectors per track for CF my Memory drive, Kingston CF 8G.

; (CPM format, 0-3D)

; Value changed to 40H for 'No Holes' LBA

--------------------------------------------------------------------

Updated disk parameter block to specify 64 sectors

--------------------------------------------------------------------

IDEHD$DPB:

;	DPB	512,32,512,2048,1024,2,8000H

;	DPB	256,32,1001,2048,1024,2,8000H

;	DPB	512,60,256,2048,1024,1,8000H

;	DPB	512,62,252,2048,1024,1,8000H

DPB	512,64,256,2048,1024,1,8000H

---------------------------------------------

Insert replacement wrlba: routine

---------------------------------------------

; Convert Track & Sector disk requests to LBA addressing & write to drive

wrlba:

LHLD	@TRK		;Get CPM requested Track Hi&Lo

MVI	H,00H		;zero high track byte

MOV	A,L		;load low track byte to accumulator

CPI	00H		;check for 0 track and skip track loop

JZ	lbasec

MVI	B,06H		;load counter to shift low track value 6 places to left i.e X 64

lbatrk:

DAD	H		;Add HL to itself 6 times to multiply by 64

DJNZ	lbatrk		;loop around 6 times i.e x 64

lbasec:

LDA	@SECT		;Get CPM requested sector

ADD	L		;Add value in L to sector info in A

JNC	lbatot		;If no carry jump to lba ‘total’ calculation

INR	H		;carry one over to H

lbatot:

MOV	L,A		;copy accumulator to L

;HL should now contain correct LBA value

MVI	D,0		;Send 0 for upper cyl value

MVI	E,REGcylinderMSB

CALL	IDEwr8D	;Send info to drive

MOV	D,H		;load lba high byte to D from H

MVI	E,REGcylinderLSB

CALL	IDEwr8D	;Send info to drive

MOV	D,L		;load lba low byte to D from L

MVI	E,REGsector

CALL	IDEwr8D	;Send info to drive

MVI	D,1		;For now, one sector at a time

MVI	E,REGseccnt

CALL	IDEwr8D

RET

==================================================================================